Table of Contents											   Page

Introduction	3

Left Side (IPv4-only Domain)	3

Right Side (Dual-Stack IPv4 & IPv6 Domain)	3

Security and Connectivity Features	3

Objectives	4

Establish a Dual-Stack Network Environment	4

Implement Secure Layer 2 Switching	4

Configure Dynamic Routing Protocols	4

Deploy Core Network Services	4

Enhance Management Security	5

Strengthen Network Defense	5

Ensure Secure Inter-Site Connectivity	5

Network Topology	6

IP Addressing Scheme	7

Global addressing summary	7

Left side (L.H.S) – IPv4 only	7

Data VLANs	7

Management VLANs (Static)	7

Inter-Router IPv4 Links	8

Right side (R.H.S) – dual stack	8

Data VLANs (IPv4 + IPv6 SLAAC)	8

Management VLANs (Dual Stack)	8

Router Links (IPv4)	8

IPv6 Tunnel Networks	9

Top right zone – ASA	9

ASA Interfaces	9

Configuration Steps	9

Switch S2	9

Switch S3	13

Switch S1	17

Router R2	21

Switch S5	24

Switch S6	29

Switch S4	33

Router R3	35

Switch S8	39

Switch S9	43

Switch S7	46

Router R4	49

Router R1	53

Switch S11	57

Switch S12	61

Switch S10	64

Router R6	67

Switch S14	72

Switch S15	75

Switch S13	78

Router R7	81

Switch S17	85

Switch S18	88

Switch S16	91

Router R8	94

Router R5	97

ASA Firewall (ASA-FW)	101

Switch S19	104

Switch S20	106

Access Point	109

Laptop	109

Tablet	111

TACACS+ Server	112

RADIUS Server	113

NTP Server	114

SysLog Server	114

DMZ Server	115

Others End Devices in Left-Side (PCs, Laptop, Tablet)	117

Others End Devices in Right-Side (PCs)	118

PC22 (Mail Settings)	118

PC28 (Mail Settings)	119

Check out the full project on GitHub	119

# Introduction

This project simulates a comprehensive enterprise network design using Cisco Packet Tracer (version 9.0.0). The objective is to design a secure, scalable, and multi-protocol network environment that integrates IPv4 and IPv6 addressing, advanced switching technologies, dynamic routing, and layered security mechanisms.

The network is divided into two major domains:

## Left Side (IPv4-only Domain)

Implements IPv4 subnetting using the 10.11.12.0/24 address space. This domain provides:

- DHCP services.

- Dynamic routing using OSPF (IPv4).

- Centralized management and security services, including NTP, Syslog, and AAA.

This side represents the internal enterprise network with strict access control and monitoring.

## Right Side (Dual-Stack IPv4 & IPv6 Domain)

Supports both IPv4 and IPv6 addressing, utilizing the 2000:12:34::/64 IPv6 prefix alongside IPv4 networks. IPv6 addressing is implemented using SLAAC. Routing within this domain relies on OSPFv2 for IPv4 only, with no OSPFv3 deployment, as IPv6 connectivity is achieved through IPv6 tunneling over an IPv4 infrastructure.

This domain hosts enterprise services, including:

- DNS Server for name resolution.

- Web Server for HTTP/HTTPS services.

- Mail Server for enterprise email communication (SMTP, POP3).

- FTP Server for file transfer.

These servers are reachable securely from the internal and external networks based on defined security policies.

## Security and Connectivity Features

To ensure strong connectivity and defense, the design incorporates:

- VLAN segmentation and Layer 2 security mechanisms such as 802.1X authentication, port security, DHCP snooping, and Dynamic ARP Inspection (DAI).

- Zone-Based Policy Firewall (ZPF) policies, Access Control Lists (ACLs), and Intrusion Prevention System (IPS) rules to enforce traffic control and protect against malicious activity.

- AAA authentication using TACACS+, RADIUS, and local authentication fallback, implementing role-based access control and customized privilege levels.

- Integration of a Cisco ASA firewall to provide DMZ protection, alongside wireless security controls for secure end-user access.

- A Site-to-Site IPSec VPN tunnel over IPv4, enabling secure communication between the two network domains.

- IPv6 tunneling across IPv4 hops, allowing end-to-end IPv6 communication through an IPv4-only routed backbone.

# Objectives

## Establish a Dual-Stack Network Environment

- Deploy IPv4 addressing (10.11.12.0/24) on the left-side network, right-side network and across inter-router links.

- Implement IPv6 addressing (2000:12:34::/64) on the right-side routers (R5–R8) and their LAN segments.

- Configure DHCP for IPv4 host address assignment.

- Configure IPv6 addressing using SLAAC.

- Enable IPv6 tunneling over IPv4, allowing IPv6 communication across IPv4-only routed hops.

## Implement Secure Layer 2 Switching

- Configure VLANs for user traffic, management, and trunk links.

- Apply Layer 2 security features including:

  - 802.1X authentication.

  - Port security.

  - DHCP snooping.

  - Dynamic ARP Inspection (DAI).

  - Spanning Tree Protocol (STP) security mechanisms, including PortFast, BPDU Guard, to prevent topology manipulation and loops.

- Isolate unused switch ports into a blackhole VLAN.

## Configure Dynamic Routing Protocols

- Deploy OSPF (IPv4) for dynamic routing on the left-side network.

- Use OSPF (IPv4) where required for IPv4 connectivity on the right side.

- IPv6 routing is achieved through tunneling mechanisms over IPv4.

- Configure static routing for networks located behind the ASA firewall.

## Deploy Core Network Services

- Configure an authenticated NTP server for left-side network time synchronization.

- Configure a Syslog server to centrally collect and store logs from the left-side network.

- Configure Router R5 on the right-side network to act as an NTP server (Master).

- Deploy enterprise application servers on the right-side network / DMZ, including:

  - DNS Server for internal and external name resolution.

  - Web Server providing HTTP and HTTPS services.

  - Mail Server supporting enterprise email communication (SMTP, POP3).

  - FTP Server for file transfer.

## Enhance Management Security

- Implement AAA authentication using TACACS+, RADIUS, and local database fallback.

- Configure role-based access control (RBAC) using privilege levels to enforce administrative separation.

## Strengthen Network Defense

- Configure a Zone-Based Policy Firewall (ZPF) on R1 to control ICMP, HTTP/HTTPS, DNS and FTP traffic.

- Deploy IOS IPS on R5 with stateful inspection, allowing traffic only when initiated from the inside network.

- Apply IPv6 ACLs on R7 to restrict unauthorized IPv6 inter-VLAN traffic.

- Configure the Cisco ASA firewall to protect the DMZ and server network by:

  - Applying security policies from INSIDE to DMZ to allow controlled internal access to servers.

  - Applying security policies from OUTSIDE to DMZ to permit only explicitly authorized public services.

- Secure wireless access using WPA2/WPA3 encryption.

## Ensure Secure Inter-Site Connectivity

- Establish a Site-to-Site IPSec VPN tunnel over IPv4 between R4 and R6.

- Enable secure inter-domain communication between network segments using encrypted IPv4 transport.

# Network Topology

# IP Addressing Scheme

## Global addressing summary

| Type | Network | Notes |
| --- | --- | --- |
| IPv4 Supernet | 10.11.12.0 /24 | Enterprise IPv4 space |
| IPv6 Global Prefix | 2000:12:34:: /64 | RHS VLANs (SLAAC) |
| IPv6 Tunnel Prefixes | 2001:12:34:X:: /64 | IPv6 over IPv4 |

## Left side (L.H.S) – IPv4 only

### Data VLANs

| VLAN | Name | Network/Mask | Gateway | DHCP Range | Broadcast |
| --- | --- | --- | --- | --- | --- |
| 10 | IT | 10.11.12.0 /29 | 10.11.12.1 | 10.11.12.3–6 | 10.11.12.7 |
| 20 | Sales | 10.11.12.8 /29 | 10.11.12.9 | 10.11.12.11–14 | 10.11.12.15 |
| 30 | Support | 10.11.12.16 /29 | 10.11.12.17 | 10.11.12.19–22 | 10.11.12.23 |
| 40 | HR | 10.11.12.24 /29 | 10.11.12.25 | 10.11.12.26–30 | 10.11.12.31 |
| 50 | Managers | 10.11.12.32 /29 | 10.11.12.33 | 10.11.12.34–38 | 10.11.12.39 |
| 60 | Developers | 10.11.12.40 /29 | 10.11.12.41 | 10.11.12.42–46 | 10.11.12.47 |
| 70 | Finance | 10.11.12.48 /29 | 10.11.12.49 | 10.11.12.50–54 | 10.11.12.55 |
| 80 | Designers | 10.11.12.56 /29 | 10.11.12.57 | 10.11.12.58–62 | 10.11.12.63 |
| 90 | Access Points | 10.11.12.64 /29 | 10.11.12.65 | 10.11.12.66–70 | 10.11.12.71 |

### Management VLANs (Static)

| VLAN | Network | Gateway | Assigned Devices |
| --- | --- | --- | --- |
| 100 | 10.11.12.72 /29 | 10.11.12.73 | Switch 1, 2, 3 |
| 110 | 10.11.12.80 /29 | 10.11.12.81 | Switch 4, 5, 6 |
| 120 | 10.11.12.88 /29 | 10.11.12.89 | Switch 7, 8, 9 |

### Inter-Router IPv4 Links

| Link | Network | Router A | Router B |
| --- | --- | --- | --- |
| R2–R1 | 10.11.12.200 /30 | 10.11.12.201 | 10.11.12.202 |
| R3–R1 | 10.11.12.204 /30 | 10.11.12.205 | 10.11.12.206 |
| R4–R1 | 10.11.12.208 /30 | 10.11.12.209 | 10.11.12.210 |
| R1–R5 | 10.11.12.212 /30 | 10.11.12.213 | 10.11.12.214 |

## Right side (R.H.S) – dual stack

### Data VLANs (IPv4 + IPv6 SLAAC)

| VLAN | Name | IPv4 Network | IPv4 GW | IPv6 Prefix | IPv6 GW |
| --- | --- | --- | --- | --- | --- |
| 200 | Instructors | 10.11.12.96 /29 | 10.11.12.97 | 2000:12:34:200::/64 | ::1 |
| 210 | Researchers | 10.11.12.104 /29 | 10.11.12.105 | 2000:12:34:210::/64 | ::1 |
| 220 | Graphics | 10.11.12.112 /29 | 10.11.12.113 | 2000:12:34:220::/64 | ::1 |
| 230 | Branding | 10.11.12.120 /29 | 10.11.12.121 | 2000:12:34:230::/64 | ::1 |
| 240 | Analysts | 10.11.12.128 /29 | 10.11.12.129 | 2000:12:34:240::/64 | ::1 |
| 250 | Auditors | 10.11.12.136 /29 | 10.11.12.137 | 2000:12:34:250::/64 | ::1 |

### Management VLANs (Dual Stack)

| VLAN | IPv4 Network | IPv4 GW | IPv6 Prefix | IPv6 GW |
| --- | --- | --- | --- | --- |
| 260 | 10.11.12.144 /29 | 10.11.12.145 | 2000:12:34:260::/64 | ::1 |
| 270 | 10.11.12.152 /29 | 10.11.12.153 | 2000:12:34:270::/64 | ::1 |
| 280 | 10.11.12.160 /29 | 10.11.12.161 | 2000:12:34:280::/64 | ::1 |

### Router Links (IPv4)

| Link | Network | Router A | Router B |
| --- | --- | --- | --- |
| R6–R5 | 10.11.12.216 /30 | 10.11.12.217 | 10.11.12.218 |
| R7–R5 | 10.11.12.220 /30 | 10.11.12.221 | 10.11.12.222 |
| R8–R5 | 10.11.12.224 /30 | 10.11.12.225 | 10.11.12.226 |

### IPv6 Tunnel Networks

| Tunnel | IPv6 Prefix | Endpoint A | Endpoint B |
| --- | --- | --- | --- |
| Tunnel 0 (R6–R7) | 2001:12:34:1::/64 | ::1 | ::2 |
| Tunnel 1 (R6–R8) | 2001:12:34:2::/64 | ::1 | ::2 |
| Tunnel 2 (R7–R8) | 2001:12:34:3::/64 | ::1 | ::2 |

## Top right zone – ASA

### ASA Interfaces

| Interface | Network | IP Address |
| --- | --- | --- |
| ASA ↔ R5 (OUTSIDE) | 10.11.12.232 /29 | 10.11.12.234 |
| ASA DMZ | 10.11.12.184 /29 | 10.11.12.185 |
| ASA INSIDE | 10.11.12.192 /29 | 10.11.12.193 |

# Configuration Steps

## Switch S2

Step 1: Enter Privileged Mode and Global Configuration

This step prepares the switch for configuration.

enable

configure terminal

Step 2: Configure Basic Device Identification and Access Security

Set the hostname, banner, local user authentication, and enable secure remote access using SSH.

hostname S2

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Configure SSH for Secure Remote Management

Enable SSH by defining a domain name and generating RSA keys.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to use local authentication and allow only SSH.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Create VLANs

Define VLANs for user traffic, management, trunking, and unused ports.

vlan 10

name IT

exit

vlan 20

name Sales

exit

vlan 30

name Support

exit

vlan 90

name Access-Point

exit

vlan 100

name Management

exit

vlan 205

name Unused-Ports

exit

vlan 201

name New-Native

exit

Step 5: Configure Access and Trunk Interfaces

Assign access ports to their respective VLANs.

interface fastEthernet 0/1

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/2

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

Configure the trunk interface with a non-default native VLAN and disable DTP.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 10,20,30,90,100

switchport trunk native vlan 201

exit

Step 6: Configure Management VLAN Interface

Assign an IP address to the management VLAN and enable the interface.

interface vlan 100

ip address 10.11.12.74 255.255.255.248

no shutdown

exit

Set the default gateway for remote management.

ip default-gateway 10.11.12.73

Step 7: Secure Unused Ports

Move unused ports to a blackhole VLAN and shut them down.

interface range fastEthernet 0/4-24, gigabitEthernet 0/2

switchport access vlan 205

shutdown

exit

Step 8: Enable Port Security on Access Ports

Restrict each access port to a single device using sticky MAC addresses.

interface range fastEthernet 0/1-3

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 9: Configure DHCP Snooping and Dynamic ARP Inspection

Protect the network against rogue DHCP servers and ARP spoofing.

ip dhcp snooping vlan 10,20,30,90

ip arp inspection vlan 10,20,30,90

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface connected to the router or DHCP server.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and enable STP protection on access ports.

interface range fastEthernet 0/1-3

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 10: Configure IEEE 802.1X Authentication

Enable AAA and configure RADIUS-based authentication for port access control.

aaa new-model

radius-server host 10.11.12.34 key cisco

aaa authentication dot1x default group radius

dot1x system-auth-control

Apply 802.1X authentication on access ports.

interface range fastEthernet 0/1-3

authentication port-control auto

dot1x pae authenticator

exit

Step 11: Configure Logging and Time Synchronization

Enable timestamped logging for accurate event tracking.

service timestamps log datetime msec

Configure Syslog to send logs to the centralized server.

logging on

logging 10.11.12.51

Configure authenticated NTP for time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 12: Save the Configuration

Ensure all changes are written to non-volatile memory.

do write

## Switch S3

Step 1: Enter Privileged Mode and Global Configuration

Prepare the switch for configuration.

enable

configure terminal

Step 2: Configure Basic Device Identification and Local Security

Set the hostname, banner, local administrator account, and enable password encryption.

hostname S3

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Configure Secure Remote Management (SSH)

Enable SSH by configuring a domain name and generating RSA keys.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to use local authentication and allow only SSH access.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Create VLANs

Define VLANs for user access, management, trunking, and unused ports.

vlan 10

name IT

exit

vlan 20

name Sales

exit

vlan 30

name Support

exit

vlan 90

name Access-Point

exit

vlan 100

name Management

exit

vlan 205

name Unused-Ports

exit

vlan 201

name New-Native

exit

Step 5: Configure Access and Trunk Interfaces

Assign access ports to their appropriate VLANs based on department requirements.

interface fastEthernet 0/1

switchport mode access

switchport access vlan 30

exit

interface fastEthernet 0/2

switchport mode access

switchport access vlan 20

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 10

exit

interface fastEthernet 0/9

switchport mode access

switchport access vlan 10

exit

Configure the trunk interface with VLAN restrictions, a dedicated native VLAN, and disable DTP.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 10,20,30,90,100

switchport trunk native vlan 201

exit

Step 6: Configure Management VLAN Interface

Assign an IP address to the management VLAN and enable the interface.

interface vlan 100

ip address 10.11.12.75 255.255.255.248

no shutdown

exit

Configure the default gateway for out-of-band management.

ip default-gateway 10.11.12.73

Step 7: Secure Unused Ports

Move all unused interfaces to a blackhole VLAN and administratively shut them down.

interface range fastEthernet 0/4-8, fastEthernet 0/10-24, gigabitEthernet 0/2

switchport access vlan 205

shutdown

exit

Step 8: Enable Port Security on Access Ports

Restrict each access port to a single dynamically learned MAC address.

interface range fastEthernet 0/1-3, fastEthernet 0/9

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 9: Configure DHCP Snooping and Dynamic ARP Inspection

Enable Layer 2 protection against rogue DHCP servers and ARP spoofing attacks.

ip dhcp snooping vlan 10,20,30,90

ip arp inspection vlan 10,20,30,90

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface connected to the router.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and STP protections on access ports.

interface range fastEthernet 0/1-3, fastEthernet 0/9

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 10: Configure IEEE 802.1X Port-Based Authentication

Enable AAA and configure RADIUS-based authentication for wired access control.

aaa new-model

radius-server host 10.11.12.34 key cisco

aaa authentication dot1x default group radius

dot1x system-auth-control

Apply 802.1X authentication to access ports.

interface range fastEthernet 0/1-3, fastEthernet 0/9

authentication port-control auto

dot1x pae authenticator

exit

Step 11: Configure Logging and Time Synchronization

Enable timestamped logging for accurate event correlation.

service timestamps log datetime msec

Send logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP for synchronized system time.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 12: Save the Configuration

Persist all configuration changes.

do write

## Switch S1

Step 1: Enter Privileged Mode and Global Configuration

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identification and Security

Define the hostname, login banner, local administrator credentials, and enable password encryption.

hostname S1

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Configure Secure Remote Management (SSH)

Enable SSH access by configuring the domain name and generating RSA keys.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to authenticate locally and accept only SSH connections.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Create VLANs

Create VLANs for user access, wireless devices, management, trunking, and unused ports.

vlan 10

name IT

exit

vlan 20

name Sales

exit

vlan 30

name Support

exit

vlan 90

name Access-Point

exit

vlan 100

name Management

exit

vlan 205

name Unused-Ports

exit

vlan 201

name New-Native

exit

Step 5: Configure Trunk and Access Interfaces

Configure trunk links to access switches and upstream devices, restricting allowed VLANs and using a non-default native VLAN.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 10,20,30,90,100

switchport trunk native vlan 201

exit

interface gigabitEthernet 0/2

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 10,20,30,90,100

switchport trunk native vlan 201

exit

interface fastEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 10,20,30,90,100

switchport trunk native vlan 201

exit

Configure the access port connected to the wireless access point.

interface fastEthernet 0/3

switchport mode access

switchport access vlan 90

exit

Step 6: Configure Management VLAN Interface

Assign an IP address to the management VLAN SVI and bring it up.

interface vlan 100

ip address 10.11.12.76 255.255.255.248

no shutdown

exit

Configure the default gateway for management traffic.

ip default-gateway 10.11.12.73

Step 7: Secure Unused Interfaces

Place all unused ports into a blackhole VLAN and shut them down to prevent unauthorized access.

interface range fastEthernet 0/2, fastEthernet 0/4-24

switchport access vlan 205

shutdown

exit

Step 8: Enable Port Security on Access Ports

Apply port security on the access port connected to the wireless access point.

interface fastEthernet 0/3

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 9: Configure DHCP Snooping and Dynamic ARP Inspection

Enable Layer 2 protection mechanisms against DHCP spoofing and ARP poisoning.

ip dhcp snooping vlan 10,20,30,90

ip arp inspection vlan 10,20,30,90

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust trunk interfaces connected to authorized DHCP sources.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Enable DHCP rate limiting and STP protections on access ports.

interface fastEthernet 0/3

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 10: Configure Logging and Time Synchronization

Enable precise log timestamps.

service timestamps log datetime msec

Send logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP for accurate time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 11: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Router R2

Step 1: Enter Privileged Mode and Global Configuration

Access privileged EXEC mode, then enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identification and Security

Set the router hostname, define a warning banner, create a local administrative user, and secure the enable mode.

hostname R2

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Configure Secure Remote Management (SSH)

Enable SSH access by configuring the domain name and generating RSA encryption keys.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to authenticate using the local user database and allow only SSH access.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Configure Inter-VLAN Routing (Router-on-a-Stick)

Create sub-interfaces on GigabitEthernet 0/0/0 to provide default gateways for each VLAN using IEEE 802.1Q encapsulation.

VLAN 10 – IT

interface gigabitEthernet 0/0/0.10

encapsulation dot1Q 10

ip address 10.11.12.1 255.255.255.248

no shutdown

exit

VLAN 20 – Sales

interface gigabitEthernet 0/0/0.20

encapsulation dot1Q 20

ip address 10.11.12.9 255.255.255.248

no shutdown

exit

VLAN 30 – Support

interface gigabitEthernet 0/0/0.30

encapsulation dot1Q 30

ip address 10.11.12.17 255.255.255.248

no shutdown

exit

VLAN 90 – Access Point

interface gigabitEthernet 0/0/0.90

encapsulation dot1Q 90

ip address 10.11.12.65 255.255.255.248

no shutdown

exit

VLAN 100 – Management

interface gigabitEthernet 0/0/0.100

encapsulation dot1Q 100

ip address 10.11.12.73 255.255.255.248

no shutdown

exit

Enable the physical interface carrying all VLAN traffic.

interface gigabitEthernet 0/0/0

no shutdown

exit

Step 5: Configure WAN Serial Interface with OSPF Authentication

Assign an IP address to the serial interface and enable OSPF message-digest authentication for secure routing updates.

interface serial 0/1/1

ip address 10.11.12.201 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 6: Configure DHCPv4 Services

Exclude statically assigned gateway addresses from DHCP allocation.

ip dhcp excluded-address 10.11.12.1

ip dhcp excluded-address 10.11.12.2

ip dhcp excluded-address 10.11.12.9

ip dhcp excluded-address 10.11.12.17

ip dhcp excluded-address 10.11.12.65

Create DHCP pools for each VLAN to automatically assign IP addresses to clients.

VLAN 10 – IT

ip dhcp pool IT-VLAN-10

network 10.11.12.0 255.255.255.248

default-router 10.11.12.1

dns-server 10.11.12.235

exit

VLAN 20 – Sales

ip dhcp pool Sales-VLAN-20

network 10.11.12.8 255.255.255.248

default-router 10.11.12.9

dns-server 10.11.12.235

exit

VLAN 30 – Support

ip dhcp pool Support-VLAN-30

network 10.11.12.16 255.255.255.248

default-router 10.11.12.17

dns-server 10.11.12.235

exit

VLAN 90 – Access Point

ip dhcp pool Access_Point-VLAN-90

network 10.11.12.64 255.255.255.248

default-router 10.11.12.65

dns-server 10.11.12.235

exit

Step 7: Configure OSPFv2 Dynamic Routing

Enable OSPF process 1, assign a router ID, and advertise all connected networks in Area 0.

router ospf 1

router-id 2.2.2.2

network 10.11.12.0 0.0.0.7 area 0

network 10.11.12.8 0.0.0.7 area 0

network 10.11.12.16 0.0.0.7 area 0

network 10.11.12.64 0.0.0.7 area 0

network 10.11.12.72 0.0.0.7 area 0

network 10.11.12.200 0.0.0.3 area 0

exit

Step 8: Configure Privilege Level

Assign a limited privilege level to support users and permit basic troubleshooting commands.

privilege exec level 5 ping

username Support privilege 5 secret support

Step 9: Configure Logging and Time Synchronization

Enable precise log timestamps for troubleshooting and auditing.

service timestamps log datetime msec

Send logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP for accurate and secure time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 10: Save the Configuration

Save the running configuration to NVRAM.

do write

## Switch S5

Step 1: Enter Privileged and Global Configuration Modes

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identification and Security

Assign a hostname, configure a warning banner, create a local administrative user, and secure privileged access.

hostname S5

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys to enable SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to use local authentication and allow only SSH access.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Create VLANs

Create and name VLANs to logically segment the network.

vlan 40

name HR

exit

vlan 50

name Managers

exit

vlan 60

name Developers

exit

vlan 110

name Management

exit

vlan 253

name Unused-Ports

exit

vlan 251

name New-Native

exit

Step 5: Configure Access Ports

Assign end-device ports to their corresponding VLANs.

interface fastEthernet 0/21

switchport mode access

switchport access vlan 40

exit

interface fastEthernet 0/10

switchport mode access

switchport access vlan 50

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 50

exit

interface fastEthernet 0/23

switchport mode access

switchport access vlan 60

exit

Step 6: Configure Trunk Port

Configure the uplink port as a trunk, restrict allowed VLANs, disable DTP negotiation, and change the native VLAN for security.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 40,50,60,110

switchport trunk native vlan 251

exit

Step 7: Configure Management VLAN Interface

Assign an IP address to the management VLAN to allow remote management of the switch.

interface vlan 110

ip address 10.11.12.82 255.255.255.248

no shutdown

exit

Step 8: Configure Default Gateway

Set the default gateway so the switch can communicate with devices outside its local subnet.

ip default-gateway 10.11.12.81

Step 9: Secure Unused Ports

Move all unused ports to a dedicated VLAN and shut them down to prevent unauthorized access.

interface range fastEthernet 0/1-2, fastEthernet 0/4-9, fastEthernet 0/11-20, fastEthernet 0/22, fastEthernet 0/24, gigabitEthernet 0/2

switchport access vlan 253

shutdown

exit

Step 10: Configure Port Security

Enable port security on user-facing access ports to limit the number of allowed MAC addresses and mitigate MAC flooding attacks.

interface range fastEthernet 0/3, fastEthernet 0/10, fastEthernet 0/21, fastEthernet 0/23

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable DHCP Snooping and Dynamic ARP Inspection

Protect the network against rogue DHCP servers and ARP spoofing attacks.

ip dhcp snooping vlan 40,50,60

ip arp inspection vlan 40,50,60

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface connected to the router or distribution switch.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and enable spanning-tree protections on access ports.

interface range fastEthernet 0/3, fastEthernet 0/10, fastEthernet 0/21, fastEthernet 0/23

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 12: Configure 802.1X Port-Based Authentication

Enable AAA, configure the RADIUS server, and activate 802.1X authentication for enhanced access control.

aaa new-model

radius-server host 10.11.12.34 key cisco

aaa authentication dot1x default group radius

dot1x system-auth-control

Apply 802.1X authentication to selected access ports.

interface range fastEthernet 0/10, fastEthernet 0/21, fastEthernet 0/23

authentication port-control auto

dot1x pae authenticator

exit

Step 13: Configure Logging and Time Synchronization

Enable detailed timestamps for log entries.

service timestamps log datetime msec

Send logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP to ensure accurate time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 14: Save the Configuration

Save the running configuration to NVRAM.

do write

## Switch S6

Step 1: Enter Privileged and Global Configuration Modes

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identification and Security

Set the hostname, configure a warning banner, create a privileged local user, and secure passwords.

hostname S6

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys to enable SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY access using local authentication and restrict remote access to SSH only.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Create VLANs

Define VLANs to segment users, management traffic, unused ports, and the native VLAN.

vlan 40

name HR

exit

vlan 50

name Managers

exit

vlan 60

name Developers

exit

vlan 110

name Management

exit

vlan 253

name Unused-Ports

exit

vlan 251

name New-Native

exit

Step 5: Configure Access Ports

Assign end-device ports to their appropriate VLANs.

interface fastEthernet 0/5

switchport mode access

switchport access vlan 60

exit

interface fastEthernet 0/3

switchport mode access

switchport access vlan 40

exit

interface fastEthernet 0/24

switchport mode access

switchport access vlan 50

exit

Step 6: Configure Trunk Port

Configure the uplink interface as a trunk, disable DTP negotiation, restrict allowed VLANs, and change the native VLAN for security.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 40,50,60,110

switchport trunk native vlan 251

exit

Step 7: Configure Management VLAN Interface

Assign an IP address to the management VLAN interface to enable remote management.

interface vlan 110

ip address 10.11.12.83 255.255.255.248

no shutdown

exit

Step 8: Configure Default Gateway

Set the default gateway to allow the switch to communicate outside its local subnet.

ip default-gateway 10.11.12.81

Step 9: Secure Unused Ports

Move all unused interfaces to a dedicated blackhole VLAN and administratively shut them down.

interface range fastEthernet 0/1-2, fastEthernet 0/4, fastEthernet 0/6-23, gigabitEthernet 0/2

switchport access vlan 253

shutdown

exit

Step 10: Configure Port Security

Enable port security on access ports to restrict devices and prevent MAC flooding attacks.

interface range fastEthernet 0/3, fastEthernet 0/5, fastEthernet 0/24

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable DHCP Snooping and Dynamic ARP Inspection

Protect the network from rogue DHCP servers and ARP spoofing attacks.

ip dhcp snooping vlan 40,50,60

ip arp inspection vlan 40,50,60

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface connected to the upstream device.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on access ports.

interface range fastEthernet 0/3, fastEthernet 0/5, fastEthernet 0/24

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 12: Configure 802.1X Authentication

Enable AAA, configure the RADIUS server, and activate port-based authentication.

aaa new-model

radius-server host 10.11.12.34 key cisco

aaa authentication dot1x default group radius

dot1x system-auth-control

Apply 802.1X authentication to user-facing access ports.

interface range fastEthernet 0/3, fastEthernet 0/5, fastEthernet 0/24

authentication port-control auto

dot1x pae authenticator

exit

Step 13: Configure Logging and Time Synchronization

Enable detailed timestamps for log messages.

service timestamps log datetime msec

Forward logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP to ensure consistent time across the network.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 14: Save the Configuration

Save the running configuration to NVRAM.

do write

## Switch S4

Step 1: Enter Privileged and Global Configuration Modes

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identification and Security

Set the hostname, configure a legal warning banner, create a privileged local user, and secure device passwords.

hostname S4

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys to enable SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to use local authentication and allow only SSH for remote access.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Create VLANs

Define VLANs for users, management, unused ports, and a non-default native VLAN to enhance security.

vlan 40

name HR

exit

vlan 50

name Managers

exit

vlan 60

name Developers

exit

vlan 110

name Management

exit

vlan 253

name Unused-Ports

exit

vlan 251

name New-Native

exit

Step 5: Configure Trunk Interfaces

Configure uplink interfaces as trunks, disable DTP negotiation, restrict allowed VLANs, and change the native VLAN to prevent VLAN hopping attacks.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 40,50,60,110

switchport trunk native vlan 251

exit

Step 6: Configure Management VLAN Interface

Assign an IP address to the management VLAN interface to allow secure remote management of the switch.

interface vlan 110

ip address 10.11.12.84 255.255.255.248

no shutdown

exit

Step 7: Configure Default Gateway

Set the default gateway so the switch can communicate with devices outside its management subnet.

ip default-gateway 10.11.12.81

Step 8: Secure Unused Ports

Move all unused interfaces to a dedicated blackhole VLAN and shut them down to reduce the attack surface.

interface range fastEthernet 0/2-24

switchport access vlan 253

shutdown

exit

Step 9: Enable DHCP Snooping and Dynamic ARP Inspection

Protect the access layer against rogue DHCP servers and ARP spoofing attacks.

ip dhcp snooping vlan 40,50,60

ip arp inspection vlan 40,50,60

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust uplink trunk interfaces connected to upstream switches or routers.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Step 10: Configure Logging and Time Synchronization

Enable precise timestamps for log messages.

service timestamps log datetime msec

Send logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP to ensure consistent time across the network.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 11: Save the Configuration

Save the running configuration to NVRAM.

do write

## Router R3

Step 1: Enter Privileged and Global Configuration Modes

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identification and Security

Set the router hostname, configure a security banner, create a privileged local user, and secure stored passwords.

hostname R3

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

enable secret cisco

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys to enable SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure console and VTY lines to use local authentication and restrict remote access to SSH only.

line vty 0 4

login local

transport input ssh

exit

line console 0

login local

exit

Step 4: Configure Router-on-a-Stick Sub-Interfaces

Create 802.1Q sub-interfaces to provide inter-VLAN routing for HR, Managers, Developers, and Management VLANs.

interface gigabitEthernet 0/0/0.40

encapsulation dot1Q 40

ip address 10.11.12.25 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/0/0.50

encapsulation dot1Q 50

ip address 10.11.12.33 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/0/0.60

encapsulation dot1Q 60

ip address 10.11.12.41 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/0/0.110

encapsulation dot1Q 110

ip address 10.11.12.81 255.255.255.248

no shutdown

exit

Enable the physical interface connected to the trunk link.

interface gigabitEthernet 0/0/0

no shutdown

exit

Step 5: Configure WAN Interface and OSPF Authentication

Assign an IP address to the serial WAN interface and enable OSPF MD5 authentication to secure routing updates.

interface serial 0/1/0

ip address 10.11.12.205 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 6: Configure DHCPv4 Services

Exclude reserved IP addresses and configure DHCP pools for VLANs served by R3.

ip dhcp excluded-address 10.11.12.25

ip dhcp excluded-address 10.11.12.33

ip dhcp excluded-address 10.11.12.34

ip dhcp excluded-address 10.11.12.40

Configure DHCP pools.

ip dhcp pool HR-VLAN-40

network 10.11.12.24 255.255.255.248

default-router 10.11.12.25

dns-server 10.11.12.235

exit

ip dhcp pool Managers-VLAN-50

network 10.11.12.32 255.255.255.248

default-router 10.11.12.33

dns-server 10.11.12.235

exit

ip dhcp pool Developers-VLAN-60

network 10.11.12.40 255.255.255.248

default-router 10.11.12.41

dns-server 10.11.12.235

exit

Step 7: Configure OSPFv2 Dynamic Routing

Enable OSPF, assign a router ID, and advertise all connected VLAN and WAN networks into Area 0.

router ospf 1

router-id 3.3.3.3

network 10.11.12.24 0.0.0.7 area 0

network 10.11.12.32 0.0.0.7 area 0

network 10.11.12.40 0.0.0.7 area 0

network 10.11.12.80 0.0.0.7 area 0

network 10.11.12.204 0.0.0.3 area 0

exit

Step 8: Configure AAA Authentication (TACACS+ and RADIUS)

Enable AAA and configure TACACS+ and RADIUS servers for centralized authentication with local fallback.

aaa new-model

tacacs-server host 10.11.12.2 single-connection key cisco

radius server SERVER-R

address ipv4 10.11.12.34 auth-port 1645

key cisco

exit

Apply the authentication method list.

aaa authentication login default group tacacs+ group radius local

Step 9: Configure Logging and Time Synchronization

Enable detailed timestamps for log messages.

service timestamps log datetime msec

Send logs to the centralized Syslog server.

logging on

logging 10.11.12.51

Configure authenticated NTP for time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 10: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S8

Step 1: Enter Privileged and Global Configuration Mode

The initial step is to access privileged EXEC mode and then enter global configuration mode to start configuring the switch.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Set the hostname, configure a login banner to warn unauthorized users, and create a local administrative user with full privileges.

hostname S8

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption to protect stored credentials.

service password-encryption

Step 3: Enable SSH for Secure Remote Management

Configure the domain name and generate RSA keys to allow secure SSH access to the switch.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines to use local authentication and allow only SSH connections.

line vty 0 4

login local

transport input ssh

exit

Configure console access to use local authentication.

line console 0

login local

exit

Set an encrypted enable password.

enable secret cisco

Step 4: Create and Name VLANs

Define VLANs to logically segment the network based on department roles and security requirements.

vlan 70

name Finance

exit

vlan 80

name Designers

exit

vlan 120

name Management

exit

vlan 276

name Unused-Ports

exit

vlan 271

name New-Native

exit

Step 5: Configure Access Ports for End Devices

Assign access ports to their respective VLANs to connect user devices.

interface fastEthernet 0/22

switchport mode access

switchport access vlan 70

exit

interface fastEthernet 0/23

switchport mode access

switchport access vlan 80

exit

Step 6: Configure Trunk Port to the Distribution Layer

Configure the trunk link to carry multiple VLANs securely and change the native VLAN for security hardening.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 70,80,120

switchport trunk native vlan 271

exit

Step 7: Configure Management VLAN Interface (SVI)

Assign an IP address to the management VLAN interface to enable remote management of the switch.

interface vlan 120

ip address 10.11.12.90 255.255.255.248

no shutdown

exit

Configure the default gateway to allow management traffic to reach other networks.

ip default-gateway 10.11.12.89

Step 8: Secure Unused Ports

Move all unused ports into a dedicated VLAN and shut them down to prevent unauthorized access.

interface range fastEthernet 0/1-21, fastEthernet 0/24, gigabitEthernet 0/2

switchport access vlan 276

shutdown

exit

Step 9: Enable Port Security on Access Ports

Apply port security to limit the number of MAC addresses and prevent unauthorized devices.

interface range fastEthernet 0/22-23

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 10: Enable DHCP Snooping and Dynamic ARP Inspection (DAI)

Enable DHCP Snooping and ARP Inspection to protect against DHCP spoofing and ARP poisoning attacks.

ip dhcp snooping vlan 70,80

ip arp inspection vlan 70,80

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Mark the trunk port as trusted for DHCP and ARP traffic.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Limit DHCP request rates and enable STP protections on access ports.

interface range fastEthernet 0/22-23

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 11: Configure IEEE 802.1X Port-Based Authentication

Enable AAA services and configure the RADIUS server for centralized authentication.

aaa new-model

radius-server host 10.11.12.34 key cisco

aaa authentication dot1x default group radius

Enable 802.1X globally on the switch.

dot1x system-auth-control

Apply 802.1X authentication to user-facing access ports.

interface range fastEthernet 0/22-23

authentication port-control auto

dot1x pae authenticator

exit

Step 12: Configure Logging and Time Synchronization

Enable timestamping on logs for accurate event tracking.

service timestamps log datetime msec

Configure a Syslog server to centralize logging.

logging on

logging 10.11.12.51

Synchronize the switch clock with an authenticated NTP server.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 13: Save the Configuration

Finally, save the running configuration to ensure persistence after a reboot.

do write

## Switch S9

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration mode to begin configuring the switch.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign a hostname to the switch, configure a security banner to warn unauthorized users, and create a local administrative account with full privileges.

hostname S9

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption to secure stored credentials.

service password-encryption

Step 3: Enable Secure Remote Management Using SSH

Define the domain name and generate RSA keys to enable SSH-based secure remote access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure the VTY lines to use local authentication and restrict remote access to SSH only.

line vty 0 4

login local

transport input ssh

exit

Configure console access to authenticate using the local user database.

line console 0

login local

exit

Set an encrypted enable secret for privileged mode access.

enable secret cisco

Step 4: Create and Name VLANs

Create VLANs to segment the network based on organizational roles and security requirements.

vlan 70

name Finance

exit

vlan 80

name Desginers

exit

vlan 120

name Management

exit

vlan 276

name Unused-Ports

exit

vlan 271

name New-Native

exit

Step 5: Configure Access Ports for End Devices

Assign selected FastEthernet ports to VLAN 70 to connect Finance department end devices.

interface range fastEthernet 0/22, fastEthernet 0/3, fastEthernet 0/1

switchport mode access

switchport access vlan 70

exit

Step 6: Configure Trunk Port to the Distribution Layer

Configure the trunk port to carry multiple VLANs securely and harden it by disabling DTP and changing the native VLAN.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 70,80,120

switchport trunk native vlan 271

exit

Step 7: Configure Management VLAN Interface (SVI)

Configure a switched virtual interface (SVI) for the management VLAN to allow remote management of the switch.

interface vlan 120

ip address 10.11.12.91 255.255.255.248

no shutdown

exit

Define the default gateway to allow management traffic to reach external networks.

ip default-gateway 10.11.12.89

Step 8: Secure Unused Switch Ports

Move all unused ports into a dedicated VLAN and administratively shut them down to prevent unauthorized access.

interface range fastEthernet 0/2, fastEthernet 0/4-21, fastEthernet 0/23-24, gigabitEthernet 0/2

switchport access vlan 276

shutdown

exit

Step 9: Enable Port Security on User Access Ports

Apply port security to restrict each access port to a single learned MAC address and define the violation behavior.

interface range fastEthernet 0/1, fastEthernet 0/3, fastEthernet 0/22

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 10: Enable DHCP Snooping and Dynamic ARP Inspection (DAI)

Enable DHCP Snooping and ARP Inspection on user VLANs to protect against DHCP spoofing and ARP poisoning attacks.

ip dhcp snooping vlan 70,80

ip arp inspection vlan 70,80

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Mark the trunk port as trusted to allow legitimate DHCP and ARP traffic.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply DHCP rate limiting and enable spanning tree protections on access ports.

interface range fastEthernet 0/1, fastEthernet 0/3, fastEthernet 0/22

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 11: Configure IEEE 802.1X Port-Based Authentication

Enable the AAA framework and configure the RADIUS server for centralized authentication.

aaa new-model

radius-server host 10.11.12.34 key cisco

aaa authentication dot1x default group radius

Enable 802.1X globally on the switch.

dot1x system-auth-control

Apply 802.1X authentication to the user-facing access ports.

interface range fastEthernet 0/1, fastEthernet 0/3, fastEthernet 0/22

authentication port-control auto

dot1x pae authenticator

exit

Step 12: Configure Logging and Time Synchronization

Enable detailed timestamps on logs to improve troubleshooting and auditing.

service timestamps log datetime msec

Configure a centralized Syslog server to collect and store logs.

logging on

logging 10.11.12.51

Synchronize system time using an authenticated NTP server.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 13: Save the Configuration

Save the running configuration to non-volatile memory to ensure persistence after a reboot.

do write

## Switch S7

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and move into global configuration mode to begin device setup.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign a hostname to uniquely identify the switch, configure a warning banner, and create a privileged local user.

hostname S7

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption to protect all stored credentials.

service password-encryption

Step 3: Enable Secure Remote Access Using SSH

Define the domain name and generate RSA keys to enable SSH for secure remote management.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines to use local authentication and allow SSH access only.

line vty 0 4

login local

transport input ssh

exit

Configure console access to authenticate using the local user database.

line console 0

login local

exit

Configure an encrypted enable secret for privileged mode access.

enable secret cisco

Step 4: Create and Name VLANs

Create VLANs to logically segment the network based on department roles and security requirements.

vlan 70

name Finance

exit

vlan 80

name Desginers

exit

vlan 120

name Management

exit

vlan 276

name Unused-Ports

exit

vlan 271

name New-Native

exit

Step 5: Configure Trunk Interfaces

Configure trunk links toward the distribution/core layer to carry multiple VLANs securely, disable DTP negotiation, and change the native VLAN.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 70,80,120

switchport trunk native vlan 271

exit

Step 6: Configure Management VLAN Interface (SVI)

Configure the switched virtual interface (SVI) for the management VLAN to allow remote management access to the switch.

interface vlan 120

ip address 10.11.12.92 255.255.255.248

no shutdown

exit

Define the default gateway to allow management traffic to reach external networks.

ip default-gateway 10.11.12.89

Step 7: Secure Unused Switch Ports

Move all unused access ports into a dedicated VLAN and shut them down to prevent unauthorized access.

interface range fastEthernet 0/2-24

switchport access vlan 276

shutdown

exit

Step 8: Enable DHCP Snooping and Dynamic ARP Inspection (DAI)

Enable DHCP Snooping and Dynamic ARP Inspection on user VLANs to protect against DHCP spoofing and ARP poisoning attacks.

ip dhcp snooping vlan 70,80

ip arp inspection vlan 70,80

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Mark trunk interfaces as trusted to allow legitimate DHCP and ARP traffic.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Step 9: Configure Logging and Time Synchronization

Enable precise timestamps for system logs to support monitoring and troubleshooting.

service timestamps log datetime msec

Configure centralized logging by forwarding logs to the Syslog server.

logging on

logging 10.11.12.51

Synchronize system time using an authenticated NTP server.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 10: Save the Configuration

Save the running configuration to ensure all settings persist after a reboot.

do write

## Router R4

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode, then enter global configuration mode to start configuring the router.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign a hostname to the router, configure a legal warning banner, and create a privileged local user account.

hostname R4

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption to secure all plaintext passwords.

service password-encryption

Step 3: Enable Secure Remote Access Using SSH

Configure the domain name and generate RSA keys to enable SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines to authenticate using the local user database and allow only SSH access.

line vty 0 4

login local

transport input ssh

exit

Configure console access to use local authentication.

line console 0

login local

exit

Configure an encrypted enable secret for privileged access.

enable secret cisco

Step 4: Configure Router-on-a-Stick Sub-Interfaces

Configure sub-interfaces on the Gigabit Ethernet interface to enable inter-VLAN routing using IEEE 802.1Q encapsulation.

interface gigabitEthernet 0/0/0.70

encapsulation dot1Q 70

ip address 10.11.12.49 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/0/0.80

encapsulation dot1Q 80

ip address 10.11.12.57 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/0/0.120

encapsulation dot1Q 120

ip address 10.11.12.89 255.255.255.248

no shutdown

exit

Enable the physical Gigabit Ethernet interface.

interface gigabitEthernet 0/0/0

no shutdown

exit

Step 5: Configure WAN Serial Interface with OSPF Authentication

Configure the serial interface for WAN connectivity and enable OSPF MD5 authentication to secure routing updates.

interface serial 0/1/0

ip address 10.11.12.209 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 6: Configure DHCPv4 Services

Exclude gateway and infrastructure addresses from DHCP allocation.

ip dhcp excluded-address 10.11.12.49

ip dhcp excluded-address 10.11.12.50

ip dhcp excluded-address 10.11.12.51

ip dhcp excluded-address 10.11.12.57

ip dhcp excluded-address 10.11.12.89

Create DHCP pools for Finance and Designers VLANs and specify their default gateways.

ip dhcp pool Finance-VLAN-70

network 10.11.12.48 255.255.255.248

default-router 10.11.12.49

dns-server 10.11.12.235

exit

ip dhcp pool Desginers-VLAN-80

network 10.11.12.56 255.255.255.248

default-router 10.11.12.57

dns-server 10.11.12.235

exit

Step 7: Enable OSPFv2 Dynamic Routing

Enable OSPFv2, assign a router ID, and advertise all connected networks into Area 0.

router ospf 1

router-id 4.4.4.4

network 10.11.12.48 0.0.0.7 area 0

network 10.11.12.56 0.0.0.7 area 0

network 10.11.12.88 0.0.0.7 area 0

network 10.11.12.208 0.0.0.3 area 0

exit

Step 8: Configure AAA Authentication

Enable AAA and configure both TACACS+ and RADIUS servers for centralized authentication with local fallback.

aaa new-model

tacacs-server host 10.11.12.2 single-connection key cisco

Configure the RADIUS server.

radius server SERVER-R

address ipv4 10.11.12.34 auth-port 1645

key cisco

exit

Apply the authentication method list.

authentication login default group radius group tacacs+ local

Step 9: Configure IPsec Site-to-Site VPN

9.1 Configure ISAKMP (IKE Phase 1) Policy

Define the ISAKMP policy parameters for secure key exchange.

crypto isakmp policy 1

group 5

authentication pre-share

hash sha

encryption aes

lifetime 3600

exit

Configure the pre-shared key for the remote peer.

crypto isakmp key cisco address 10.11.12.217

9.2 Define Interesting Traffic Using an Access Control List

Specify the networks that should be encrypted through the VPN tunnel.

ip access-list extended VPN-Connection

permit ip 10.11.12.48 0.0.0.7 10.11.12.96 0.0.0.7

permit ip 10.11.12.48 0.0.0.7 10.11.12.104 0.0.0.7

permit ip 10.11.12.48 0.0.0.7 10.11.12.144 0.0.0.7

permit ip 10.11.12.56 0.0.0.7 10.11.12.96 0.0.0.7

permit ip 10.11.12.56 0.0.0.7 10.11.12.104 0.0.0.7

permit ip 10.11.12.56 0.0.0.7 10.11.12.144 0.0.0.7

permit ip 10.11.12.88 0.0.0.7 10.11.12.96 0.0.0.7

permit ip 10.11.12.88 0.0.0.7 10.11.12.104 0.0.0.7

permit ip 10.11.12.88 0.0.0.7 10.11.12.144 0.0.0.7

exit

9.3 Configure IPsec Phase 2 Parameters

Define the IPsec transform set.

crypto ipsec transform-set R4-R6 esp-aes

Create and configure the crypto map.

crypto map R4-R6_MAP 10 ipsec-isakmp

match address VPN-Connection

set transform-set R4-R6

set peer 10.11.12.217

set pfs group5

set security-association lifetime seconds 900

exit

Apply the crypto map to the WAN interface.

interface serial 0/1/0

crypto map R4-R6_MAP

exit

Step 10: Configure Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure centralized Syslog logging.

logging on

logging 10.11.12.51

Configure authenticated NTP for time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 11: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Router R1

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the hostname, configure a warning banner, and create a privileged local user.

hostname R1

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption for all plaintext passwords.

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Restrict VTY access to SSH only and authenticate using the local user database.

line vty 0 4

login local

transport input ssh

exit

Configure console access authentication.

line console 0

login local

exit

Configure an encrypted enable secret.

enable secret cisco

Step 4: Configure WAN Serial Interfaces with OSPF Authentication

Configure all serial interfaces connecting to other routers. Each interface uses IP addressing and OSPF MD5 authentication to protect routing updates.

interface serial 0/1/1

ip address 10.11.12.202 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

interface serial 0/2/1

ip address 10.11.12.210 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

interface serial 0/2/0

ip address 10.11.12.206 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

interface serial 0/1/0

ip address 10.11.12.213 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 5: Enable OSPFv2 Dynamic Routing

Enable OSPFv2, assign a unique router ID, and advertise all WAN networks into Area 0.

router ospf 1

router-id 1.1.1.1

network 10.11.12.200 0.0.0.3 area 0

network 10.11.12.208 0.0.0.3 area 0

network 10.11.12.204 0.0.0.3 area 0

network 10.11.12.212 0.0.0.3 area 0

exit

Step 6: Configure Zone-Based Policy Firewall (ZPF)

Create security zones to separate trusted internal traffic from external traffic.

zone security IN

exit

zone security OUT

exit

Step 7: Define Traffic Classification for Firewall Policies

Create a class map to match common inside-to-outside protocols.

class-map type inspect match-any CLASS_IN2OUT

match protocol icmp

match protocol http

match protocol https

match protocol ftp

match protocol dns

exit

Create an access control list to allow IPsec-related traffic.

ip access-list extended ACL_IPsec

permit udp any any eq 500

permit udp any any eq 4500

permit esp any any

exit

Match the IPsec traffic using a class map.

class-map type inspect match-any CLASS-IPsec

match access-group name ACL_IPsec

exit

Step 8: Create Zone-Based Firewall Policies

Define the policy for traffic flowing from the inside zone to the outside zone.

policy-map type inspect POLICY_IN2OUT

class CLASS-IPsec

pass

class CLASS_IN2OUT

inspect

exit

exit

Define the policy for traffic flowing from the outside zone to the inside zone.

policy-map type inspect POLICY_OUT2IN

class CLASS-IPsec

pass

exit

exit

Step 9: Apply Firewall Policies Using Zone Pairs

Bind the security policies to zone pairs.

zone-pair security PAIR_IN2OUT source IN destination OUT

service-policy type inspect POLICY_IN2OUT

exit

zone-pair security PAIR_OUT2IN source OUT destination IN

service-policy type inspect POLICY_OUT2IN

exit

Step 10: Assign Interfaces to Security Zones

Assign each serial interface to its appropriate security zone.

interface serial 0/1/1

zone-member security IN

exit

interface serial 0/2/0

zone-member security IN

exit

interface serial 0/2/1

zone-member security IN

exit

interface serial 0/1/0

zone-member security OUT

exit

Step 11: Configure Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure centralized Syslog logging.

logging on

logging 10.11.12.51

Configure authenticated NTP to ensure accurate time synchronization.

ntp server 10.11.12.50

ntp authenticate

ntp trusted-key 1

ntp authentication-key 1 md5 cisco

Step 12: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S11

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the hostname, configure a warning banner, and create a privileged local user.

hostname S11

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption to secure stored credentials.

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys required for SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Restrict VTY access to SSH only and authenticate using the local user database.

line vty 0 4

login local

transport input ssh

exit

Enable console access authentication.

line console 0

login local

exit

Configure an encrypted enable secret.

enable secret cisco

Step 4: Create VLANs for Network Segmentation

Create VLANs to logically separate user groups, management traffic, unused ports, and the native VLAN.

vlan 200

name Instructors

exit

vlan 210

name Researchers

exit

vlan 260

name Management

exit

vlan 300

name Unused-Ports

exit

vlan 305

name New-Native

exit

Step 5: Configure Access and Trunk Ports

Assign access ports to their respective VLANs.

interface fastEthernet 0/23

switchport mode access

switchport access vlan 200

exit

interface fastEthernet 0/20

switchport mode access

switchport access vlan 210

exit

Configure the trunk port with a restricted VLAN list and a secure native VLAN.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 200,210,260

switchport trunk native vlan 305

exit

Step 6: Enable IPv6 Support (Dual-Stack Operation)

Change the SDM template to support IPv4 and IPv6, save the configuration, and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

After reload, enable IPv6 routing globally.

ipv6 unicast-routing

Step 7: Configure Management VLAN with IPv4 and IPv6

Assign both IPv4 and IPv6 addresses to the management VLAN interface.

interface vlan 260

ip address 10.11.12.146 255.255.255.248

ipv6 address 2000:12:34:260::2/64

ipv6 enable

no shutdown

exit

Step 8: Configure Default Gateways

Configure the IPv6 default route for management traffic.

ipv6 route ::/0 2000:12:34:260::1

Configure the IPv4 default gateway for switch management.

ip default-gateway 10.11.12.145

Step 9: Isolate and Disable Unused Ports

Move all unused interfaces to a dedicated blackhole VLAN and shut them down.

interface range fastEthernet 0/1-19, fastEthernet 0/21-22, fastEthernet 0/24, gigabitEthernet 0/2

switchport access vlan 300

shutdown

exit

Step 10: Enable Port Security on Access Ports

Restrict each access port to a single learned MAC address using sticky MAC learning.

interface range fastEthernet 0/20, fastEthernet 0/23

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection (DAI) to protect against spoofing attacks.

ip dhcp snooping vlan 200,210

ip arp inspection vlan 200,210

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface connected to the router or core switch.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Step 12: Apply STP Protection and Rate Limiting

Enable PortFast, BPDU Guard, and limit DHCP request rates on access ports.

interface range fastEthernet 0/20, fastEthernet 0/23

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 13: Configure Logging and Time Synchronization

Enable timestamps for system logs.

service timestamps log datetime msec

Configure NTP for accurate time synchronization.

ntp server 10.11.12.218

Step 14: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S12

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration mode.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Set the switch hostname, configure a legal warning banner, and create a privileged local user.

hostname S12

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Restrict VTY access to SSH and authenticate using the local database.

line vty 0 4

login local

transport input ssh

exit

Enable console authentication.

line console 0

login local

exit

Configure an encrypted enable password.

enable secret cisco

Step 4: Create VLANs

Create VLANs for user segmentation, management, unused ports, and a secure native VLAN.

vlan 200

name Instructors

exit

vlan 210

name Researchers

exit

vlan 260

name Management

exit

vlan 300

name Unused-Ports

exit

vlan 305

name New-Native

exit

Step 5: Configure Access and Trunk Interfaces

Assign access ports to their respective VLANs.

interface fastEthernet 0/22

switchport mode access

switchport access vlan 200

exit

interface fastEthernet 0/21

switchport mode access

switchport access vlan 210

exit

Configure the trunk interface with VLAN restrictions and disable DTP.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 200,210,260

switchport trunk native vlan 305

exit

Step 6: Enable Dual-Stack IPv4/IPv6 Support

Change the SDM template to support both IPv4 and IPv6, then save and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

After reload, enable IPv6 routing.

ipv6 unicast-routing

Step 7: Configure Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management SVI.

interface vlan 260

ip address 10.11.12.147 255.255.255.248

ipv6 address 2000:12:34:260::3/64

ipv6 enable

no shutdown

exit

Step 8: Configure Default Gateways

Configure the IPv6 default route.

ipv6 route ::/0 2000:12:34:260::1

Configure the IPv4 default gateway.

ip default-gateway 10.11.12.145

Step 9: Secure Unused Interfaces

Move all unused ports to a dedicated VLAN and shut them down.

interface range fastEthernet 0/1-20, fastEthernet 0/23-24, gigabitEthernet 0/2

switchport access vlan 300

shutdown

exit

Step 10: Enable Port Security on Access Ports

Limit each access port to a single learned MAC address using sticky learning.

interface range fastEthernet 0/21-22

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable Layer 2 Protection Mechanisms

Enable DHCP Snooping and Dynamic ARP Inspection for user VLANs.

ip dhcp snooping vlan 200,210

ip arp inspection vlan 200,210

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface connected to the distribution layer.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Step 12: Apply Spanning Tree and Rate Limiting Protections

Enable PortFast, BPDU Guard, and limit DHCP request rates on access ports.

interface range fastEthernet 0/21-22

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 13: Enable Logging and Time Synchronization

Enable timestamps on system logs.

service timestamps log datetime msec

Configure NTP for accurate time synchronization.

ntp server 10.11.12.218

Step 14: Save the Configuration

Save the running configuration to NVRAM.

do write

## Switch S10

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and move to global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Set the hostname, configure a warning banner, and create a privileged local user.

hostname S10

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Restrict VTY access to SSH using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable console authentication.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create VLANs for users, management, unused ports, and a secure native VLAN.

vlan 200

name Instructors

exit

vlan 210

name Researchers

exit

vlan 260

name Management

exit

vlan 300

name Unused-Ports

exit

vlan 305

name New-Native

exit

Step 5: Configure Trunk Interfaces

Configure uplink and inter-switch links as trunks, disable DTP negotiation, restrict allowed VLANs, and assign a non-default native VLAN.

interface range gigabitEthernet 0/1, fastEthernet 0/1, fastEthernet 0/24

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 200,210,260

switchport trunk native vlan 305

exit

Step 6: Enable Dual-Stack IPv4/IPv6 Support

Modify the SDM template to support IPv4 and IPv6, then save and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

After reload, enable IPv6 routing.

ipv6 unicast-routing

Step 7: Configure Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management SVI.

interface vlan 260

ip address 10.11.12.148 255.255.255.248

ipv6 address 2000:12:34:260::4/64

ipv6 enable

no shutdown

exit

Step 8: Enable Layer 2 Security Mechanisms

Enable DHCP Snooping and Dynamic ARP Inspection for user VLANs.

ip dhcp snooping vlan 200,210

ip arp inspection vlan 200,210

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Mark trunk interfaces as trusted for DHCP Snooping and DAI.

interface range gigabitEthernet 0/1, fastEthernet 0/1, fastEthernet 0/2

ip dhcp snooping trust

ip arp inspection trust

exit

Step 9: Configure Default Gateways

Configure the IPv6 default route toward the distribution router.

ipv6 route ::/0 2000:12:34:260::1

Configure the IPv4 default gateway.

ip default-gateway 10.11.12.145

Step 10: Secure Unused Interfaces

Move all unused ports to a dedicated VLAN and shut them down.

interface range fastEthernet 0/2-23, gigabitEthernet 0/2

switchport access vlan 300

shutdown

exit

Step 11: Enable Logging and Time Synchronization

Enable timestamping on system logs.

service timestamps log datetime msec

Configure NTP to synchronize time with the centralized time server.

ntp server 10.11.12.218

Step 12: Save the Configuration

Save the running configuration to NVRAM.

do write

## Router R6

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Set the router hostname, configure a security warning banner, and create a privileged local user.

hostname R6

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Management (SSH)

Configure the domain name and generate RSA keys for SSH access.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Restrict VTY access to SSH using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable console authentication.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Enable IPv6 Routing

Activate IPv6 routing to support dual-stack operation.

ipv6 unicast-routing

Step 5: Configure Router-on-a-Stick Subinterfaces

Configure subinterfaces for VLANs 200, 210, and 260 with both IPv4 and IPv6 addressing.

interface gigabitEthernet 0/0/0.200

encapsulation dot1Q 200

ip address 10.11.12.97 255.255.255.248

ipv6 address 2000:12:34:200::1/64

ipv6 enable

no shutdown

exit

interface gigabitEthernet 0/0/0.210

encapsulation dot1Q 210

ip address 10.11.12.105 255.255.255.248

ipv6 address 2000:12:34:210::1/64

ipv6 enable

no shutdown

exit

interface gigabitEthernet 0/0/0.260

encapsulation dot1Q 260

ip address 10.11.12.145 255.255.255.248

ipv6 address 2000:12:34:260::1/64

ipv6 enable

no shutdown

exit

Enable the physical interface.

interface gigabitEthernet 0/0/0

no shutdown

exit

Step 6: Configure Serial WAN Interface

Assign an IPv4 address and enable OSPF MD5 authentication on the serial link.

interface serial 0/1/0

ip address 10.11.12.217 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 7: Configure IPv6-over-IPv4 Tunnel to R7

Create an IPv6 tunnel over IPv4 to reach R7 via R5.

interface Tunnel0

ipv6 address 2001:12:34:1::1/64

tunnel source Se0/1/0

tunnel destination 10.11.12.221

tunnel mode ipv6ip

exit

Add static IPv6 routes to R7’s LANs through the tunnel.

ipv6 route 2000:12:34:220::/64 2001:12:34:1::2

ipv6 route 2000:12:34:230::/64 2001:12:34:1::2

ipv6 route 2000:12:34:270::/64 2001:12:34:1::2

Step 8: Configure IPv6-over-IPv4 Tunnel to R8

Create a second IPv6 tunnel to reach R8 via R5.

interface Tunnel1

ipv6 address 2001:12:34:2::1/64

tunnel source Se0/1/0

tunnel destination 10.11.12.225

tunnel mode ipv6ip

exit

Add static IPv6 routes to R8’s LANs through the tunnel.

ipv6 route 2000:12:34:240::/64 2001:12:34:2::2

ipv6 route 2000:12:34:250::/64 2001:12:34:2::2

ipv6 route 2000:12:34:280::/64 2001:12:34:2::2

Step 9: Enable OSPFv2 Routing

Configure OSPF with a unique router ID and advertise all connected networks.

router ospf 1

router-id 6.6.6.6

network 10.11.12.96 0.0.0.7 area 0

network 10.11.12.104 0.0.0.7 area 0

network 10.11.12.144 0.0.0.7 area 0

network 10.11.12.216 0.0.0.3 area 0

exit

Step 10: Configure DHCPv4 Services

Exclude gateway IP addresses from DHCP allocation.

ip dhcp excluded-address 10.11.12.97

ip dhcp excluded-address 10.11.12.105

Create DHCP pools for VLAN 200 and VLAN 210.

ip dhcp pool Instructors-VLAN-200

network 10.11.12.96 255.255.255.248

default-router 10.11.12.97

dns-server 10.11.12.235

exit

ip dhcp pool Researchers-VLAN-210

network 10.11.12.104 255.255.255.248

default-router 10.11.12.105

dns-server 10.11.12.235

exit

Step 11: Configure Role-Based CLI Views

Enable AAA and activate role-based views.

aaa new-model

exit

enable view

Create a read-only monitoring view.

parser view SHOW

secret show

commands exec include all show

exit

Create a restricted DHCP configuration view.

parser view DHCP_Conf

secret dhcp

commands exec include configure terminal

command configure include all ip dhcp

command configure include network

command configure include default-router

command configure include dns-server

command configure include exit

command configure include no

commands exec include all show ip dhcp

exit

Step 12: Configure IPsec VPN with R4

Create an ISAKMP policy and define the pre-shared key.

crypto isakmp policy 1

group 5

authentication pre-share

hash sha

encryption aes

lifetime 3600

exit

crypto isakmp key cisco address 10.11.12.209

Define interesting traffic for the VPN.

ip access-list extended VPN-Connection

permit ip 10.11.12.96 0.0.0.7 10.11.12.48 0.0.0.7

permit ip 10.11.12.96 0.0.0.7 10.11.12.56 0.0.0.7

permit ip 10.11.12.96 0.0.0.7 10.11.12.88 0.0.0.7

permit ip 10.11.12.104 0.0.0.7 10.11.12.48 0.0.0.7

permit ip 10.11.12.104 0.0.0.7 10.11.12.56 0.0.0.7

permit ip 10.11.12.104 0.0.0.7 10.11.12.88 0.0.0.7

permit ip 10.11.12.144 0.0.0.7 10.11.12.48 0.0.0.7

permit ip 10.11.12.144 0.0.0.7 10.11.12.56 0.0.0.7

permit ip 10.11.12.144 0.0.0.7 10.11.12.88 0.0.0.7

exit

Configure IPsec transform set and crypto map.

crypto ipsec transform-set R4-R6 esp-aes

crypto map R4-R6_MAP 10 ipsec-isakmp

match address VPN-Connection

set transform-set R4-R6

set peer 10.11.12.209

set pfs group5

set security-association lifetime seconds 900

exit

Apply the crypto map to the WAN interface.

interface serial 0/1/0

crypto map R4-R6_MAP

exit

Step 13: Enable Logging and Time Synchronization

Enable timestamps on system logs.

service timestamps log datetime msec

Configure NTP synchronization.

ntp server 10.11.12.218

Step 14: Save the Configuration

Save the running configuration to NVRAM.

do write

## Switch S14

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and move to global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the hostname, configure a warning banner, and create a privileged local user.

hostname S14

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Define the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines to allow SSH access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create and name all required VLANs, including user, management, and unused-port VLANs.

vlan 220

name Graphics

exit

vlan 230

name Branding

exit

vlan 270

name Management

exit

vlan 330

name Unused-Ports

exit

vlan 335

name New-Native

exit

Step 5: Configure Access and Trunk Ports

Assign access ports to their respective VLANs.

interface fastEthernet 0/22

switchport mode access

switchport access vlan 220

exit

interface fastEthernet 0/20

switchport mode access

switchport access vlan 230

exit

Configure the trunk interface with a restricted VLAN list and a new native VLAN.

interface gigabitEthernet 0/2

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 220,230,270

switchport trunk native vlan 335

exit

Step 6: Enable IPv6 Support (SDM Template)

Modify the SDM template to support IPv6 and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

Step 7: Configure Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management SVI and enable it.

interface vlan 270

ip address 10.11.12.154 255.255.255.248

ipv6 address 2000:12:34:270::2/64

ipv6 enable

no shutdown

exit

Enable IPv6 routing.

ipv6 unicast-routing

Step 8: Configure Default Gateways

Configure both IPv4 and IPv6 default gateways for management connectivity.

ip default-gateway 10.11.12.153

ipv6 route ::/0 2000:12:34:270::1

Step 9: Secure Unused Ports

Assign all unused ports to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/1-19, fastEthernet 0/21, fastEthernet 0/23, fastEthernet 0/24, gigabitEthernet 0/1

switchport access vlan 330

shutdown

exit

Step 10: Enable Port Security on Access Ports

Restrict each access port to a single learned MAC address using sticky MAC addressing.

interface range fastEthernet 0/20, fastEthernet 0/22

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection for user VLANs.

ip dhcp snooping vlan 220,230

ip arp inspection vlan 220,230

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface.

interface gigabitEthernet 0/2

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on access ports.

interface range fastEthernet 0/20, fastEthernet 0/22

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 12: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP synchronization.

ntp server 10.11.12.222

Step 13: Save the Configuration

Save the running configuration to NVRAM.

do write

## Switch S15

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S15

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create and name all required VLANs, including user, management, and unused-port VLANs.

vlan 220

name Graphics

exit

vlan 230

name Branding

exit

vlan 270

name Management

exit

vlan 330

name Unused-Ports

exit

vlan 335

name New-Native

exit

Step 5: Configure Access and Trunk Ports

Assign access ports to their corresponding VLANs.

interface fastEthernet 0/24

switchport mode access

switchport access vlan 220

exit

interface fastEthernet 0/19

switchport mode access

switchport access vlan 230

exit

Configure the trunk interface, restrict allowed VLANs, disable DTP negotiation, and assign a new native VLAN.

interface gigabitEthernet 0/2

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 220,230,270

switchport trunk native vlan 335

exit

Step 6: Enable IPv6 Support (SDM Template)

Change the SDM template to support IPv6 operation and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

Step 7: Configure the Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management VLAN interface and enable it.

interface vlan 270

ip address 10.11.12.155 255.255.255.248

ipv6 address 2000:12:34:270::3/64

ipv6 enable

no shutdown

exit

Enable IPv6 routing on the switch.

ipv6 unicast-routing

Step 8: Configure Default Gateways

Configure both IPv4 and IPv6 default gateways to allow management traffic to reach external networks.

ip default-gateway 10.11.12.153

ipv6 route ::/0 2000:12:34:270::1

Step 9: Secure Unused Ports

Move all unused interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/1-18, fastEthernet 0/20-23, gigabitEthernet 0/1

switchport access vlan 330

shutdown

exit

Step 10: Enable Port Security on Access Ports

Limit access ports to a single MAC address using sticky MAC learning and restrict violations.

interface range fastEthernet 0/19, fastEthernet 0/24

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection on user VLANs.

ip dhcp snooping vlan 220,230

ip arp inspection vlan 220,230

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface.

interface gigabitEthernet 0/2

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on access ports.

interface range fastEthernet 0/19, fastEthernet 0/24

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 12: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.222

Step 13: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S13

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S13

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create and name all required VLANs, including user, management, and unused-port VLANs.

vlan 220

name Graphics

exit

vlan 230

name Branding

exit

vlan 270

name Management

exit

vlan 330

name Unused-Ports

exit

vlan 335

name New-Native

exit

Step 5: Configure Trunk Ports

Configure trunk interfaces, restrict allowed VLANs, disable DTP negotiation, and assign a new native VLAN.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 220,230,270

switchport trunk native vlan 335

exit

Step 6: Enable IPv6 Support (SDM Template)

Change the SDM template to support IPv6 operation and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

Step 7: Configure the Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management VLAN interface and enable it.

interface vlan 270

ip address 10.11.12.156 255.255.255.248

ipv6 address 2000:12:34:270::4/64

ipv6 enable

no shutdown

exit

Enable IPv6 routing on the switch.

ipv6 unicast-routing

Step 8: Configure Default Gateways

Configure both IPv4 and IPv6 default gateways to allow management traffic to reach external networks.

ip default-gateway 10.11.12.153

ipv6 route ::/0 2000:12:34:270::1

Step 9: Secure Unused Ports

Move all unused interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/2-24

switchport access vlan 330

shutdown

exit

Step 10: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection on user VLANs.

ip dhcp snooping vlan 220,230

ip arp inspection vlan 220,230

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust trunk interfaces for DHCP and ARP inspection.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Step 11: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.222

Step 12: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Router R7

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the router hostname, configure a security banner, and create a privileged local user.

hostname R7

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Enable IPv6 Routing

Enable IPv6 routing globally on the router.

ipv6 unicast-routing

Step 5: Configure Router-on-a-Stick Subinterfaces

Configure VLAN subinterfaces with IPv4 and IPv6 addressing.

Graphics VLAN (220)

interface gigabitEthernet 0/0/0.220

encapsulation dot1Q 220

ip address 10.11.12.113 255.255.255.248

ipv6 address 2000:12:34:220::1/64

ipv6 enable

no shutdown

exit

Branding VLAN (230)

interface gigabitEthernet 0/0/0.230

encapsulation dot1Q 230

ip address 10.11.12.121 255.255.255.248

ipv6 address 2000:12:34:230::1/64

ipv6 enable

no shutdown

exit

Management VLAN (270)

interface gigabitEthernet 0/0/0.270

encapsulation dot1Q 270

ip address 10.11.12.153 255.255.255.248

ipv6 address 2000:12:34:270::1/64

ipv6 enable

no shutdown

exit

Enable the physical interface.

interface gigabitEthernet 0/0/0

no shutdown

exit

Step 6: Configure WAN Serial Interface

Configure the serial interface and enable OSPF authentication.

interface serial 0/1/0

ip address 10.11.12.221 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 7: Configure IPv6-over-IPv4 Tunnel to Router R6

Create a tunnel interface to carry IPv6 traffic over the IPv4 network.

interface Tunnel0

ipv6 address 2001:12:34:1::2/64

tunnel source Se0/1/0

tunnel destination 10.11.12.217

tunnel mode ipv6ip

exit

Configure IPv6 static routes for R6 networks via the tunnel.

ipv6 route 2000:12:34:200::/64 2001:12:34:1::1

ipv6 route 2000:12:34:210::/64 2001:12:34:1::1

ipv6 route 2000:12:34:260::/64 2001:12:34:1::1

Step 8: Configure IPv6-over-IPv4 Tunnel to Router R8

Create a tunnel interface toward R8.

interface Tunnel2

ipv6 address 2001:12:34:3::1/64

tunnel source Se0/1/0

tunnel destination 10.11.12.225

tunnel mode ipv6ip

exit

Configure IPv6 static routes for R8 networks via the tunnel.

ipv6 route 2000:12:34:240::/64 2001:12:34:3::2

ipv6 route 2000:12:34:250::/64 2001:12:34:3::2

ipv6 route 2000:12:34:280::/64 2001:12:34:3::2

Step 9: Enable OSPFv2 Routing

Configure OSPFv2 for IPv4 routing.

router ospf 1

router-id 7.7.7.7

network 10.11.12.112 0.0.0.7 area 0

network 10.11.12.120 0.0.0.7 area 0

network 10.11.12.152 0.0.0.7 area 0

network 10.11.12.220 0.0.0.3 area 0

exit

Step 10: Configure DHCPv4 Services

Exclude gateway addresses from DHCP.

ip dhcp excluded-address 10.11.12.113

ip dhcp excluded-address 10.11.12.121

Create DHCP pool for Graphics VLAN.

ip dhcp pool Graphics-VLAN-220

network 10.11.12.112 255.255.255.248

default-router 10.11.12.113

dns-server 10.11.12.235

exit

Create DHCP pool for Branding VLAN.

ip dhcp pool Branding-VLAN-230

network 10.11.12.120 255.255.255.248

default-router 10.11.12.121

dns-server 10.11.12.235

exit

Step 11: Configure IPv6 Access Control List

Create an IPv6 ACL to control Branding VLAN traffic.

ipv6 access-list Branding2Out

permit icmp 2000:12:34:230::0/64 any nd-na

permit icmp 2000:12:34:230::0/64 any nd-ns

permit icmp 2000:12:34:230::0/64 any echo-reply

exit

Apply the IPv6 ACL inbound on the Branding subinterface.

interface gigabitEthernet 0/0/0.230

ipv6 traffic-filter Branding2Out in

exit

Step 12: Enable Logging and Time Synchronization

Enable timestamps for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.222

Step 13: Save the Configuration

Save the running configuration.

do write

## Switch S17

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S17

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create and name all required VLANs, including user, management, and unused-port VLANs.

vlan 240

name Analysts

exit

vlan 250

name Auditors

exit

vlan 280

name Management

exit

vlan 360

name Unused-Ports

exit

vlan 365

name New-Native

exit

Step 5: Configure Access and Trunk Ports

Assign access ports to their corresponding VLANs.

Analysts VLAN (240)

interface fastEthernet 0/23

switchport mode access

switchport access vlan 240

exit

Auditors VLAN (250)

interface fastEthernet 0/21

switchport mode access

switchport access vlan 250

exit

Configure the trunk interface, restrict allowed VLANs, disable DTP negotiation, and assign a new native VLAN.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 240,250,280

switchport trunk native vlan 365

exit

Step 6: Enable IPv6 Support (SDM Template)

Change the SDM template to support IPv6 operation and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

Step 7: Configure the Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management VLAN interface and enable it.

interface vlan 280

ip address 10.11.12.162 255.255.255.248

ipv6 address 2000:12:34:280::2/64

ipv6 enable

no shutdown

exit

Enable IPv6 routing on the switch.

ipv6 unicast-routing

Step 8: Configure Default Gateways

Configure both IPv4 and IPv6 default gateways for management traffic.

ip default-gateway 10.11.12.161

ipv6 route ::/0 2000:12:34:280::1

Step 9: Secure Unused Ports

Move all unused interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/1-20, fastEthernet 0/22, fastEthernet 0/24, gigabitEthernet 0/2

switchport access vlan 360

shutdown

exit

Step 10: Enable Port Security on Access Ports

Restrict access ports to a single MAC address using sticky MAC learning.

interface range fastEthernet 0/21, fastEthernet 0/23

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection on user VLANs.

ip dhcp snooping vlan 240,250

ip arp inspection vlan 240,250

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on access ports.

interface range fastEthernet 0/21, fastEthernet 0/23

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 12: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.226

Step 13: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S18

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S18

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create and name all required VLANs, including user, management, and unused-port VLANs.

vlan 240

name Analysts

exit

vlan 250

name Auditors

exit

vlan 280

name Management

exit

vlan 360

name Unused-Ports

exit

vlan 365

name New-Native

exit

Step 5: Configure Access and Trunk Ports

Assign access ports to their corresponding VLANs.

Analysts VLAN (240)

interface fastEthernet 0/21

switchport mode access

switchport access vlan 240

exit

Auditors VLAN (250)

interface fastEthernet 0/22

switchport mode access

switchport access vlan 250

exit

Configure the trunk interface, restrict allowed VLANs, disable DTP negotiation, and assign a new native VLAN.

interface gigabitEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 240,250,280

switchport trunk native vlan 365

exit

Step 6: Enable IPv6 Support (SDM Template)

Change the SDM template to support IPv6 operation and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

Step 7: Configure the Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management VLAN interface and enable it.

interface vlan 280

ip address 10.11.12.163 255.255.255.248

ipv6 address 2000:12:34:280::3/64

ipv6 enable

no shutdown

exit

Enable IPv6 routing on the switch.

ipv6 unicast-routing

Step 8: Configure Default Gateways

Configure both IPv4 and IPv6 default gateways for management traffic.

ip default-gateway 10.11.12.161

ipv6 route ::/0 2000:12:34:280::1

Step 9: Secure Unused Ports

Move all unused interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/1-20, fastEthernet 0/23-24, gigabitEthernet 0/2

switchport access vlan 360

shutdown

exit

Step 10: Enable Port Security on Access Ports

Restrict access ports to a single MAC address using sticky MAC learning.

interface range fastEthernet 0/21-22

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 11: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection on user VLANs.

ip dhcp snooping vlan 240,250

ip arp inspection vlan 240,250

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the trunk interface.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on access ports.

interface range fastEthernet 0/21-22

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 12: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.226

Step 13: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S16

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S16

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create and name all required VLANs, including user, management, and unused-port VLANs.

vlan 240

name Analysts

exit

vlan 250

name Auditors

exit

vlan 280

name Management

exit

vlan 360

name Unused-Ports

exit

vlan 365

name New-Native

exit

Step 5: Configure Trunk Interfaces

Configure all uplink interfaces as trunks, restrict allowed VLANs, disable DTP negotiation, and assign a new native VLAN.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

switchport mode trunk

switchport nonegotiate

switchport trunk allowed vlan 240,250,280

switchport trunk native vlan 365

exit

Step 6: Enable IPv6 Support (SDM Template)

Change the SDM template to support IPv6 operation and reload the switch.

sdm prefer dual-ipv4-and-ipv6 default

do write

do reload

Step 7: Configure the Management VLAN Interface

Assign IPv4 and IPv6 addresses to the management VLAN interface and enable it.

interface vlan 280

ip address 10.11.12.164 255.255.255.248

ipv6 address 2000:12:34:280::4/64

ipv6 enable

no shutdown

exit

Enable IPv6 routing on the switch.

ipv6 unicast-routing

Step 8: Configure Default Gateways

Configure both IPv4 and IPv6 default gateways for management traffic.

ip default-gateway 10.11.12.161

ipv6 route ::/0 2000:12:34:280::1

Step 9: Secure Unused Ports

Move all unused access interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/2-24

switchport access vlan 360

shutdown

exit

Step 10: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection on user VLANs.

ip dhcp snooping vlan 240,250

ip arp inspection vlan 240,250

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust trunk interfaces connected to infrastructure devices.

interface range gigabitEthernet 0/1-2, fastEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Step 11: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.226

Step 12: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Router R8

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the router hostname, configure a security banner, and create a privileged local user.

hostname R8

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Enable IPv6 Routing

Enable IPv6 packet forwarding on the router.

ipv6 unicast-routing

Step 5: Configure Router-on-a-Stick Subinterfaces

Configure subinterfaces on GigabitEthernet0/0/0 to provide inter-VLAN routing for IPv4 and IPv6.

VLAN 240 – Analysts

interface gigabitEthernet 0/0/0.240

encapsulation dot1Q 240

ip address 10.11.12.129 255.255.255.248

ipv6 address 2000:12:34:240::1/64

ipv6 enable

no shutdown

exit

VLAN 250 – Auditors

interface gigabitEthernet 0/0/0.250

encapsulation dot1Q 250

ip address 10.11.12.137 255.255.255.248

ipv6 address 2000:12:34:250::1/64

ipv6 enable

no shutdown

exit

VLAN 280 – Management

interface gigabitEthernet 0/0/0.280

encapsulation dot1Q 280

ip address 10.11.12.161 255.255.255.248

ipv6 address 2000:12:34:280::1/64

ipv6 enable

no shutdown

exit

Enable the physical interface.

interface gigabitEthernet 0/0/0

no shutdown

exit

Step 6: Configure WAN Serial Interface

Assign an IPv4 address to the serial interface and enable OSPF MD5 authentication.

interface serial 0/1/1

ip address 10.11.12.225 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

Step 7: Configure IPv6-over-IPv4 Tunnel to R6

Create a tunnel interface to provide IPv6 connectivity to R6 through an IPv4-only path.

interface Tunnel1

ipv6 address 2001:12:34:2::2/64

tunnel source Se0/1/1

tunnel destination 10.11.12.217

tunnel mode ipv6ip

exit

Add static IPv6 routes for R6’s LANs through the tunnel.

ipv6 route 2000:12:34:200::/64 2001:12:34:2::1

ipv6 route 2000:12:34:210::/64 2001:12:34:2::1

ipv6 route 2000:12:34:260::/64 2001:12:34:2::1

Step 8: Configure IPv6-over-IPv4 Tunnel to R7

Create a second tunnel interface to provide IPv6 connectivity to R7.

interface Tunnel2

ipv6 address 2001:12:34:3::2/64

tunnel source Se0/1/1

tunnel destination 10.11.12.221

tunnel mode ipv6ip

exit

Add static IPv6 routes for R7’s LANs through the tunnel.

ipv6 route 2000:12:34:220::/64 2001:12:34:3::1

ipv6 route 2000:12:34:230::/64 2001:12:34:3::1

ipv6 route 2000:12:34:270::/64 2001:12:34:3::1

Step 9: Enable OSPFv2 Routing

Configure OSPFv2 for IPv4 routing and advertise all connected networks.

router ospf 1

router-id 8.8.8.8

network 10.11.12.128 0.0.0.7 area 0

network 10.11.12.136 0.0.0.7 area 0

network 10.11.12.160 0.0.0.7 area 0

network 10.11.12.224 0.0.0.3 area 0

exit

Step 10: Configure DHCPv4 Services

Exclude gateway addresses from DHCP allocation.

ip dhcp excluded-address 10.11.12.129

ip dhcp excluded-address 10.11.12.137

Create a DHCP pool for the Analysts VLAN.

ip dhcp pool Analysts-VLAN-240

network 10.11.12.128 255.255.255.248

default-router 10.11.12.129

dns-server 10.11.12.235

exit

Create a DHCP pool for the Auditors VLAN.

ip dhcp pool Auditors-VLAN-250

network 10.11.12.136 255.255.255.248

default-router 10.11.12.137

dns-server 10.11.12.235

exit

Step 11: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

Configure NTP for time synchronization.

ntp server 10.11.12.226

Step 12: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Router R5

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the router hostname, configure a security banner, and create a privileged local user.

hostname R5

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Configure WAN and LAN Interfaces

Assign IPv4 addresses to all serial and Gigabit interfaces and enable OSPF MD5 authentication on WAN links.

Serial Interfaces

interface serial 0/1/0

ip address 10.11.12.214 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

interface serial 0/1/1

ip address 10.11.12.218 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

interface serial 0/2/0

ip address 10.11.12.222 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

interface serial 0/2/1

ip address 10.11.12.226 255.255.255.252

ip ospf authentication message-digest

ip ospf message-digest-key 1 md5 cisco

no shutdown

exit

GigabitEthernet Interface

interface gigabitEthernet 0/0/0

ip address 10.11.12.233 255.255.255.248

no shutdown

exit

Loopback Interface (Router ID Stability)

interface loopback 0

ip address 5.5.5.5 255.255.255.0

no shutdown

exit

Step 5: Enable OSPFv2 Routing

Configure OSPFv2, set a stable router ID, and advertise all connected networks.

router ospf 1

router-id 5.5.5.5

network 10.11.12.212 0.0.0.3 area 0

network 10.11.12.216 0.0.0.3 area 0

network 10.11.12.220 0.0.0.3 area 0

network 10.11.12.224 0.0.0.3 area 0

network 10.11.12.228 0.0.0.3 area 0

network 10.11.12.232 0.0.0.3 area 0

exit

Step 6: Configure Static Routes for Inside Hosts

Add host-specific static routes for internal devices.

ip route 10.11.12.235 255.255.255.255 10.11.12.234

ip route 10.11.12.237 255.255.255.255 10.11.12.234

Step 7: Enable IOS Intrusion Prevention System (IPS)

Create a local directory for IPS files and define an IPS policy.

mkdir ipsdir

configure terminal

ip ips config location ipsdir

ip ips name iosips

Enable only the basic IOS IPS signature category.

ip ips signature-category

category all

retired true

exit

category ios_ips basic

retired false

exit

exit

Apply the IPS policy outbound on WAN interfaces.

interface serial 0/1/0

ip ips iosips in

exit

interface gigabitEthernet 0/0/0

ip ips iosips in

exit

Step 8: Configure IPS ICMP Inspection

Enable IPS inspection for ICMP traffic and drop malicious packets inline.

ip ips signature-definition

signature 2004 0

status

retired false

enabled true

exit

engine

event-action deny-packet-inline

event-action produce-alert

exit

exit

exit

Step 9: Enable Logging and Time Synchronization

Enable timestamping for system logs.

service timestamps log datetime msec

exit

Manually set the system clock and configure the router as an NTP master.

clock set 19:14:50 jan 31 2025

configure terminal

ntp master 1

Step 10: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## ASA Firewall (ASA-FW)

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the firewall hostname, domain name, enable password, and create a local user.

hostname ASA-FW

domain-name cisco.local

enable password cisco

username cisco password cisco

Generate RSA keys for SSH access.

crypto key generate rsa modulus 1024

yes

Enable SSH access from the INSIDE network using local authentication.

ssh 10.11.12.192 255.255.255.248 INSIDE

aaa authentication ssh console LOCAL

Step 3: Configure Firewall Interfaces

Configure all security zones with appropriate security levels and IP addressing.

OUTSIDE Interface

interface gigabitEthernet 1/1

nameif OUTSIDE

security-level 0

ip address 10.11.12.234 255.255.255.248

no shutdown

exit

INSIDE Interface

interface gigabitEthernet 1/2

nameif INSIDE

security-level 100

ip address 10.11.12.193 255.255.255.248

no shutdown

exit

DMZ Interface

interface gigabitEthernet 1/3

nameif DMZ

security-level 50

ip address 10.11.12.185 255.255.255.248

no shutdown

exit

Step 4: Enable Application Inspection

Allow stateful inspection for common protocols to permit return traffic.

policy-map global_policy

class inspection_default

inspect dns

no inspect icmp

no inspect ftp

no inspect tftp

no inspect http

exit

Step 5: Configure Default Route

Define a default route toward the upstream router on the OUTSIDE interface.

route OUTSIDE 0.0.0.0 0.0.0.0 10.11.12.233

Step 6: Configure Network Objects and NAT

Create network objects and configure static NAT for INSIDE and DMZ resources.

INSIDE Network Static NAT

object network INSIDE-Network

subnet 10.11.12.192 255.255.255.248

nat (INSIDE,OUTSIDE) static 10.11.12.237

exit

DMZ Network Object

object network DMZ-Network

subnet 10.11.12.184 255.255.255.248

exit

DMZ Server Static NAT

object network SERVER-DMZ-NET

host 10.11.12.187

nat (DMZ,OUTSIDE) static 10.11.12.235

exit

DMZ PC Static NAT

object network PC-DMZ-NET

host 10.11.12.188

nat (DMZ,OUTSIDE) static 10.11.12.236

exit

Step 7: Configure Access Control Lists (ACLs)

OUTSIDE Interface ACL

Allow external users to access specific services hosted in the DMZ.

access-list OUTSIDE_ACL extended permit tcp any host 10.11.12.235 eq 80

access-list OUTSIDE_ACL extended permit tcp any host 10.11.12.235 eq 443

access-list OUTSIDE_ACL extended permit tcp any host 10.11.12.235 eq 25

access-list OUTSIDE_ACL extended permit tcp any host 10.11.12.235 eq 110

access-list OUTSIDE_ACL extended permit udp any host 10.11.12.235 eq 53

access-list OUTSIDE_ACL extended permit tcp any host 10.11.12.235 eq 21

access-list OUTSIDE_ACL extended permit tcp any host 10.11.12.235 eq 20

access-list OUTSIDE_ACL extended permit tcp host 10.11.12.235 eq 443 host 10.11.12.237

access-list OUTSIDE_ACL extended permit tcp host 10.11.12.235 eq www host 10.11.12.237

access-list OUTSIDE_ACL extended permit icmp any any echo-reply

access-group OUTSIDE_ACL in interface OUTSIDE

DMZ Interface ACL

Allow INSIDE users to access DMZ services using management protocols.

access-list DMZ_ACL extended permit tcp object DMZ-Network object INSIDE-Network eq 22

access-list DMZ_ACL extended permit tcp object DMZ-Network object INSIDE-Network eq 23

access-list DMZ_ACL extended permit udp host 10.11.12.187 eq 53 any

access-group DMZ_ACL in interface DMZ

Step 8: Save the Configuration

Save the running configuration to non-volatile memory.

write memory

## Switch S19

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S19

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create a dedicated VLAN for unused ports.

vlan 500

name Unused-Ports

exit

Step 5: Configure Access Ports

Configure active user ports as access ports.

interface range fastEthernet 0/1-2

switchport mode access

exit

Step 6: Configure the Management VLAN Interface

Assign an IP address to the default VLAN interface for management access.

interface vlan 1

ip address 10.11.12.186 255.255.255.248

no shutdown

exit

Step 7: Configure Default Gateway

Define the default gateway for management traffic.

ip default-gateway 10.11.12.185

Step 8: Secure Unused Ports

Move all unused interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/3-24, gigabitEthernet 0/2

switchport access vlan 500

shutdown

exit

Step 9: Enable Port Security on Access Ports

Restrict access ports to a single MAC address using sticky MAC learning.

interface range fastEthernet 0/1-2

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 10: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection.

ip dhcp snooping

ip arp inspection vlan 1

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the uplink interface.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on access ports.

interface range fastEthernet 0/1-2

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 11: Enable Logging

Enable timestamping for system logs.

service timestamps log datetime msec

Step 12: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Switch S20

Step 1: Enter Privileged and Global Configuration Mode

Access privileged EXEC mode and enter global configuration.

enable

configure terminal

Step 2: Configure Basic Device Identity and Security

Assign the switch hostname, configure a security banner, and create a privileged local user.

hostname S20

banner motd &#### Authorized Access Only ####&

username cisco privilege 15 secret cisco

Enable password encryption.

service password-encryption

Step 3: Enable Secure Remote Access (SSH)

Configure the domain name and generate RSA keys required for SSH.

ip domain-name cisco.local

crypto key generate rsa general-keys modulus 1024

Configure VTY lines for SSH-only access using local authentication.

line vty 0 4

login local

transport input ssh

exit

Enable authentication on the console line.

line console 0

login local

exit

Configure a secure enable password.

enable secret cisco

Step 4: Create VLANs

Create a dedicated VLAN for unused ports.

vlan 510

name Unused-Ports

exit

Step 5: Configure Access Ports

Configure the active user port as an access port.

interface fastEthernet 0/1

switchport mode access

exit

Step 6: Configure the Management VLAN Interface

Assign an IP address to the default VLAN interface for management access.

interface vlan 1

ip address 10.11.12.194 255.255.255.248

no shutdown

exit

Step 7: Configure Default Gateway

Define the default gateway for management traffic.

ip default-gateway 10.11.12.193

Step 8: Secure Unused Ports

Move all unused interfaces to a dedicated VLAN and administratively shut them down.

interface range fastEthernet 0/2-24, gigabitEthernet 0/2

switchport access vlan 510

shutdown

exit

Step 9: Enable Port Security on Access Ports

Restrict the access port to a single MAC address using sticky MAC learning.

interface fastEthernet 0/1

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

exit

Step 10: Enable Layer 2 Security Features

Enable DHCP Snooping and Dynamic ARP Inspection.

ip dhcp snooping

ip arp inspection vlan 1

ip arp inspection validate src-mac

ip arp inspection validate dst-mac

ip arp inspection validate ip

Trust the uplink interface.

interface gigabitEthernet 0/1

ip dhcp snooping trust

ip arp inspection trust

exit

Apply rate limiting and spanning-tree protections on the access port.

interface fastEthernet 0/1

ip dhcp snooping limit rate 6

spanning-tree portfast

spanning-tree bpduguard enable

exit

Step 11: Enable Logging

Enable timestamping for system logs.

service timestamps log datetime msec

Step 12: Save the Configuration

Save the running configuration to non-volatile memory.

do write

## Access Point

## Laptop

- Power OFF the laptop.

- Add Wireless Card (WPC300N).

- Power ON the laptop.

## Tablet

## TACACS+ Server

## RADIUS Server

## NTP Server

## SysLog Server

## DMZ Server

## Others End Devices in Left-Side (PCs, Laptop, Tablet)

## Others End Devices in Right-Side (PCs)

## PC22 (Mail Settings)

## PC28 (Mail Settings)

Notes

- All Servers have static IPs.

- Hardening All servers.

- DMZ and INSIDE zones have static IPs.

- When INSIDE wants to reach web server in DMZ, they reach it from OUTSIDE not directly from INSIDE to DMZ.

# Check out the full project on GitHub

https://github.com/M0oham6d/Enterprise-Network-Design-and-Security-Implementation.git

Thanks